Task 1:

;*****************************************************************

;* Lab 9 - Task 1: PWM0 ? Buzzer (1 kHz, 50% duty)               *

;* MCU: MC9S12DT256                                              *

;* LEDs: PB4–PB7                                                 *

;*****************************************************************

XDEF   Entry, _Startup

ABSENTRY Entry

INCLUDE 'derivative.inc'

ROMStart    EQU    $4000

;---------------------------------------------------------

; Variable/data section

;---------------------------------------------------------

ORG    RAMStart

Counter     DS.W   1

DelayCount  DS.W   1

;---------------------------------------------------------

; Code section

;---------------------------------------------------------

ORG    ROMStart

Entry:

_Startup:

LDS    #RAMEnd+1              ; Initialize stack pointer

CLI                           ; Enable interrupts (not used here)

MOVB #$10, PWME               ; PWME 4 Enable

MOVB #$00, PWMCLK             ; Send 0 to Clock A

MOVB #$03, PWMPRCLK           ; Multiplier by 8.

MOVB #$10, PWMPOL             ; Link PWME to channel 4

MOVB #$0C, PWMCTL

CLR PWMCNT4 ;CLEAR 4TH CHANNEL

Loop:      MOVB #250, PWMPER4 ; We need period of 1ms = 250 * 8 (multiplier in PWMPOL) * 0.5 (bus clock)

MOVB #125, PWMDTY4 ; We need a duty cycle which is 50% of period

BRA Loop

;---------------------------------------------------------

; Interrupt Vector

;---------------------------------------------------------

ORG    $FFFE

DC.W   Entry

Task 2:

Part a:

;*****************************************************************

;* Lab 9 - Task 2-a            *

;* MCU: MC9S12DT256                                              *

;* LEDs: PB4–PB7                                                 *

;*****************************************************************

XDEF   Entry, _Startup

ABSENTRY Entry

INCLUDE 'derivative.inc'

ROMStart    EQU    $4000

;---------------------------------------------------------

; Variable/data section

;---------------------------------------------------------

ORG    RAMStart

Counter     DS.W   1

DelayCount  DS.W   1

;---------------------------------------------------------

; Code section

;---------------------------------------------------------

ORG    ROMStart

Entry:

_Startup:

LDS    #RAMEnd+1              ; Initialize stack pointer

CLI                           ; Enable interrupts (not used here)

MOVB #$10, PWME               ; PWME 4 Enable

MOVB #$00, PWMCLK             ; Send 0 to Clock A

MOVB #$03, PWMPRCLK           ; Multiplier by 8.

MOVB #$10, PWMPOL             ; Link PWME to channel 4

MOVB #$0C, PWMCTL

CLR PWMCNT4 ;CLEAR 4TH CHANNEL

CLR PWMCNT0 ; Clear PWM0 counter

CLR PWMCNT1 ; Clear 	PWM1 counter

Loop:

MOVW #426, PWMPER0 ; Period = 426 × 8 × 0.5μs = 1704μs ≈ 587Hz 	MOVW #426, PWMPER1 ; Same period for PWM1

MOVW #298, PWMDTY0 ; PWM0 duty = 298/426 = 69.95% ≈ 70% 	MOVW #298, PWMDTY1 ; PWM1 duty = 298/426 = 69.95% ≈ 70%

BRA Loop

;---------------------------------------------------------

; Interrupt Vector

;---------------------------------------------------------

ORG $FFFE

DC.W Entry

Part b:

Change dury cycle to:

MOVW #128, PWMDTY1           ; PWM1 (LED): 30% duty (0.30 × 426= 128)

Part c:

Experimental Observations:

When running Task 2a (70% duty cycle on PWM5/LED):

- The LED appears bright

- The LED is ON for 70% of the time (HIGH for 1193μs, LOW for 511μs per cycle)

- Average voltage/power delivered to LED ≈ 70% of maximum

When running Task 2b (30% duty cycle on PWM5/LED):

- The LED appears dim

- The LED is ON for only 30% of the time (HIGH for 511μs, LOW for 1193μs per cycle)

- Average voltage/power delivered to LED ≈ 30% of maximum

Conclusion:

- The duty cycle directly controls the brightness of the LED:

  - Higher Duty Cycle → Brighter LED

  - 70% duty cycle = LED appears at approximately 70% brightness

  - More time spent in HIGH state = more average power

- Lower Duty Cycle → Dimmer LED

  - 30% duty cycle = LED appears at approximately 30% brightness

  - Less time spent in HIGH state = less average power

- Why this works:

  - At 587Hz, the LED switches ON/OFF 587 times per second

  - This switching is too fast for the human eye to detect (persistence of vision)

  - The eye perceives the average brightness based on the duty cycle

  - The LED current is also proportional to the duty cycle