Design and Implementation of a 3-Floor Elevator Control System
Using Freescale HC12 Microcontroller

COE324 - Microcontroller Laboratory
Final Project Report

Submitted by:
Rawad Abou Ibrahim: 202206158

Rim Al Aridi: 202208898

Safi Al Aridi: 202304421

06-12-2025

Department of Computer and Communications Engineering
Lebanese American University

Abstract

This report presents the design, implementation, and analysis of a three-floor elevator control system developed for the Freescale HC12 microcontroller platform. The system demonstrates real-time embedded systems programming principles through the integration of multiple hardware peripherals including analog-to-digital conversion (ADC), pulse-width modulation (PWM), serial peripheral interface (SPI), and general-purpose input/output (GPIO) ports. The elevator system features floor navigation, weight-based overload protection, visual and audio feedback mechanisms, and an intelligent priority-based routing algorithm. The implementation utilizes interrupt-driven architecture with a 100ms periodic interrupt for sensor monitoring and button polling. The system successfully demonstrates the application of embedded systems concepts including state machine design, hardware interfacing, and real-time control algorithms.

Keywords: Embedded Systems, Microcontroller, HC12, Elevator Control, Real-Time Systems, State Machine

1. Introduction

1.1 Background

Elevator control systems represent a classic application of embedded systems engineering, requiring real-time response, safety-critical operation, and integration of multiple hardware peripherals. Modern elevator systems must handle multiple floor requests, prioritize operations efficiently, and provide user feedback through visual and audio indicators. This project implements a simplified but functionally complete elevator control system on the Freescale HC12 microcontroller platform.

1.2 Objectives

The primary objectives of this project are:

System Design: Develop a complete 3-floor elevator control system with floor navigation capabilities

Hardware Integration: Interface multiple peripherals including ADC, PWM, SPI, and GPIO

Safety Implementation: Implement weight-based overload detection and prevention mechanisms

User Interface: Provide visual (LEDs, LCD) and audio (buzzer) feedback to users

Algorithm Development: Design and implement an intelligent floor selection algorithm

Real-Time Operation: Ensure responsive system behavior through interrupt-driven architecture

1.3 Scope

This project focuses on the software implementation of elevator control logic using assembly language for the HC12 microcontroller. The system supports three floors (F0, F1, F2), handles both internal and external call buttons, monitors weight through ADC, and provides comprehensive user feedback. The implementation demonstrates embedded systems programming principles including interrupt handling, state machine design, and hardware register manipulation.

2. System Overview

2.1 System Architecture

The elevator control system follows a hierarchical architecture consisting of:

Main Control Loop: Manages high-level state transitions and movement decisions

Interrupt Service Routine (ISR): Handles periodic tasks including sensor reading and button polling

Hardware Abstraction Layer: Subroutines for LCD, buzzer, and delay operations

State Machine: Defines system states (IDLE, UP, DOWN, OVERLOADED)

2.2 Functional Requirements

The system must satisfy the following functional requirements:

Floor Navigation: Move between three floors (F0, F1, F2) based on user requests

Button Handling: Process both internal (inside elevator) and external (hallway) call buttons

Overload Detection: Monitor weight sensor and prevent operation when overloaded

Visual Feedback: Display current floor and movement status on LCD

LED Indicators: Use colored LEDs to indicate movement direction and system state

Audio Feedback: Provide buzzer alerts for door operations and overload conditions

Priority Routing: Select destination floor based on proximity and request priority

2.3 Non-Functional Requirements

Real-Time Response: System must respond to button presses within 100ms

Safety: Overload condition must immediately halt elevator movement

Reliability: System must handle multiple simultaneous requests correctly

User Experience: Clear visual and audio feedback for all operations

3. Hardware Design

3.1 Microcontroller Platform

The system is implemented on the MC9S12DT256 microcontroller, a member of the Freescale HC12 family with the following specifications:

CPU: 16-bit HCS12 core running at 8MHz (E-clock)

Memory: 256KB Flash, 12KB RAM

Peripherals:

8-channel 8/10-bit ADC

8-channel PWM module

SPI interface

Modulus Counter (MCCNT)

Multiple GPIO ports

3.2 Input Components

3.2.1 Push Buttons (Port A)

Configuration: 6 buttons total (3 internal + 3 external)

Port: PORTA, bits 2-7 (PA2-PA7)

Logic: Active-low with pull-up resistors enabled via PUCR register

Mapping:

PA2 (PB4): Internal F0 request

PA3 (PB5): Internal F1 request

PA4 (PB6): Internal F2 request

PA5 (PB7): External F0 request

PA6 (PB8): External F1 request

PA7 (PB9): External F2 request

3.2.2 Weight Sensor (ADC Channel 5)

Type: Potentiometer-based weight simulation

ADC Channel: Channel 5 (ATD0DR0L)

Resolution: 8-bit (0-255)

Threshold: 125 (overload condition)

Configuration: Single conversion mode, right-justified result

3.3 Output Components

3.3.1 LED Indicators (Port T)

Green LED (PT7): Indicates upward movement

Yellow LED (PT6): Indicates downward movement

Red LED (PT5): Indicates arrival or overload condition

Configuration: Output mode via DDRT register, active-high logic

3.3.2 Buzzer (PWM Channel 4)

Port: Port P, Pin 4 (PP4)

PWM Channel: Channel 4

Frequency: ~587Hz (period = 250 clock cycles, prescaler = E/8)

Duty Cycle: 50% (125/250)

Control: Enabled/disabled via PWME register

3.3.3 LCD Display (SPI Interface)

Type: HD44780-compatible 16x2 character LCD

Interface: SPI (Serial Peripheral Interface)

Port: Port M (SPI data lines)

Configuration: Master mode, CPOL=1, CPHA=0

Display Modes:

Current floor (F0, F1, F2)

Movement status (MOVING UP:, MOVING DOWN:)

Overload warning (OVERLOAD)

3.4 Interrupt System

3.4.1 Modulus Counter (MCCNT)

Purpose: Generate periodic interrupts for system monitoring

Period: 100ms (6250 clock cycles at 8MHz with prescaler)

Interrupt Vector: $FFCA

Function: Triggers ISR for ADC reading and button polling

4. Software Architecture

4.1 Program Structure

The software is organized into the following sections:

Initialization Section: Hardware setup and variable initialization

Main Loop: Primary control logic and state management

Interrupt Service Routine: Periodic sensor reading and button handling

Movement Routines: Up and down movement control

Subroutines: LCD, buzzer, and delay functions

4.2 Memory Organization

ROMStart: $4000          ; Code section start
RAMStart: (from derivative.inc)  ; Variable section

Variables:
- Current:    Current floor (0, 1, or 2)
- Target:     Target floor (0, 1, 2, or 3 for idle)
- State:      System state (0=idle, 1=up, 2=down, 3=overload)
- ADCValue:   Latest ADC reading
- Overload:   Overload flag (0=normal, 1=overloaded)
- Request0-2: Floor request flags
- BlinkCount: LED blink counter

4.3 State Machine Design

The system operates as a finite state machine with four primary states:

State 0: IDLE

Description: Waiting for floor requests

Actions: Monitor buttons, select target floor

Transitions:

→ State 1 (UP): When target > current

→ State 2 (DOWN): When target < current

→ State 3 (OVERLOADED): When weight exceeds threshold

State 1: MOVING UP

Description: Elevator moving upward

Actions:

Blink green LED

Display "MOVING UP:" on LCD

Increment current floor counter

Transitions:

→ State 0 (IDLE): When current == target

→ State 3 (OVERLOADED): If overload detected

State 2: MOVING DOWN

Description: Elevator moving downward

Actions:

Blink yellow LED

Display "MOVING DOWN:" on LCD

Decrement current floor counter

Transitions:

→ State 0 (IDLE): When current == target

→ State 3 (OVERLOADED): If overload detected

State 3: OVERLOADED

Description: Weight limit exceeded

Actions:

Blink red LED and buzzer

Display "OVERLOAD" on LCD

Prevent all movement

Transitions:

→ State 0 (IDLE): When weight returns to normal

4.4 Priority Algorithm

The system implements a proximity-based priority algorithm for floor selection:

From Floor 0 (F0):

Priority 1: Floor 1 (if requested)

Priority 2: Floor 2 (if requested)

From Floor 1 (F1):

Priority 1: Closest requested floor (F0 or F2)

If both requested: Serve F0 first (downward priority)

From Floor 2 (F2):

Priority 1: Floor 1 (if requested)

Priority 2: Floor 0 (if requested)

This algorithm minimizes travel distance and provides efficient service to multiple requests.

5. Implementation Details

5.1 Initialization Sequence

The system initialization follows this sequence:

Stack Pointer Setup: Initialize stack to RAMEnd+1

Variable Initialization: Clear all state variables, set initial floor to F0

Port Configuration:

Port A: Input mode for buttons

Port T: Output mode for LEDs

Port P: Output mode for buzzer

Port M: SPI interface configuration

PWM Initialization: Configure buzzer PWM channel

ADC Initialization: Setup ADC for weight sensor reading

MCCNT Setup: Configure modulus counter for 100ms interrupts

SPI Configuration: Setup SPI for LCD communication

LCD Initialization: Send initialization commands to LCD

5.2 Main Control Loop

The main loop implements the following logic:

MainLoop:
1. Check overload condition
   - If overloaded → goto OverloadWait
2. Check if target is set (Target != 3)
   - If no target → loop back
3. Compare current and target floors
   - If equal → clear target, loop back
   - If target > current → goto MOVEUP
   - If target < current → goto MOVEDOWN

5.3 Interrupt Service Routine (MCCNT_ISR)

The ISR executes every 100ms and performs:

ADC Reading:

Trigger ADC conversion on channel 5

Wait for conversion complete

Read result and store in ADCValue

Compare with threshold (125)

Set/clear Overload flag

Button Polling:

Read PORTA register

Invert logic (active-low)

Check each button bit

Set corresponding Request flag

Target Selection:

If target is idle (Target == 3)

Based on current floor, select highest priority requested floor

Set Target variable

5.4 Movement Routines

5.4.1 MOVEUP Routine

Set State = 1

Play door closing beep (long beep)

Display "MOVING UP:" on LCD

Loop:

Blink green LED (10 cycles)

Increment Current floor

Check if arrived at target

If not, repeat loop

When arrived → goto ARRIVED

5.4.2 MOVEDOWN Routine

Set State = 2

Play door closing beep (long beep)

Display "MOVING DOWN:" on LCD

Loop:

Blink yellow LED (10 cycles)

Decrement Current floor

Check if arrived at target

If not, repeat loop

When arrived → goto ARRIVED

5.4.3 ARRIVED Routine

Turn off movement LEDs

Turn on red LED and buzzer (door open signal)

Wait for door open delay (2 seconds)

Update LCD with current floor

Clear request for current floor

Reset Target and State

Turn off red LED and buzzer

Return to MainLoop

5.5 Overload Handling

The overload handling routine (OverloadWait) performs:

Display "OVERLOAD" message on LCD

Blink red LED and buzzer synchronously

Continuously check Overload flag

When weight normalizes:

Turn off red LED and buzzer

Clear overload state

Restore normal floor display

Return to MainLoop

5.6 Buzzer Control

The buzzer system provides three distinct audio signals:

Door Closing Beep: Long beep (~2 seconds) before movement

Arrival Signal: Buzzer ON with red LED when door opens

Overload Warning: Synchronized blinking with red LED

PWM Configuration:

Period: 250 clock cycles (~587Hz)

Duty Cycle: 50% (125/250)

Prescaler: E/8 (1MHz clock)

5.7 LCD Communication

LCD communication uses SPI interface with the following protocol:

Instruction Mode: RS=0, sends control commands

Data Mode: RS=1, sends character data

4-bit Mode: Data sent in two 4-bit nibbles

Timing: Delay between commands for LCD processing

Key LCD operations:

Clear display

Set cursor position

Send character data

Display status messages

6. Testing and Results

6.1 Test Scenarios

Test 1: Basic Floor Navigation

Objective: Verify elevator moves correctly between floors

Procedure:

Start at F0

Press button for F2

Observe movement sequence

Expected Results:

Green LED blinks during upward movement

LCD displays "MOVING UP:"

Elevator arrives at F2

Red LED and buzzer activate on arrival

LCD displays "F2"

Result: ✅ PASS - All expected behaviors observed

Test 2: Priority Algorithm

Objective: Verify floor selection prioritizes closest floor

Procedure:

Start at F0

Press buttons for both F1 and F2

Observe which floor is served first

Expected Results:

Elevator serves F1 first (closer)

Then proceeds to F2

Result: ✅ PASS - Priority algorithm functions correctly

Test 3: Overload Detection

Objective: Verify overload protection prevents movement

Procedure:

Set weight sensor above threshold (125)

Attempt to request floor

Observe system response

Expected Results:

Red LED blinks

Buzzer sounds

LCD displays "OVERLOAD"

Elevator does not move

When weight reduced, system returns to normal

Result: ✅ PASS - Overload protection works as designed

Test 4: Button Handling

Objective: Verify both internal and external buttons function

Procedure:

Test each internal button (PA2-PA4)

Test each external button (PA5-PA7)

Verify requests are registered

Expected Results:

All buttons register requests correctly

No false triggers observed

Result: ✅ PASS - All buttons function correctly

Test 5: Simultaneous Requests

Objective: Verify system handles multiple requests correctly

Procedure:

Press multiple floor buttons

Observe elevator serves requests in priority order

Expected Results:

System queues requests

Serves floors based on priority algorithm

Clears requests after service

Result: ✅ PASS - Multiple requests handled correctly

6.2 Performance Metrics

Interrupt Response Time: < 100ms (deterministic)

Button Response Time: < 100ms (polled in ISR)

Overload Detection Time: < 100ms (checked every interrupt)

LCD Update Time: ~50ms per character

Movement Simulation: 10 blink cycles per floor (~2 seconds)

6.3 System Limitations

No Emergency Stop: System lacks emergency stop functionality

Fixed Timing: Movement timing is fixed, not based on actual position

Simple Priority: Priority algorithm is basic; could be enhanced

No Door Sensor: Door open/close is simulated with delays

Single Elevator: System designed for single elevator only

7. Discussion

7.1 Design Decisions

7.1.1 Interrupt-Driven Architecture

The choice to use periodic interrupts (MCCNT) rather than polling in the main loop provides several advantages:

Deterministic Timing: Sensor readings occur at fixed intervals

Efficiency: Main loop can focus on high-level control

Responsiveness: Button presses detected within 100ms

7.1.2 State Machine Design

The state machine approach provides:

Clear Logic Flow: Easy to understand and debug

Predictable Behavior: Each state has well-defined transitions

Maintainability: Easy to add new states or modify behavior

7.1.3 Assembly Language Implementation

Using assembly language offers:

Direct Hardware Control: Precise register manipulation

Performance: No compiler overhead

Educational Value: Deep understanding of microcontroller operation

7.2 Challenges Encountered

Timing Synchronization: Coordinating LED blinking, buzzer, and LCD updates required careful delay management

Button Debouncing: Active-low logic with pull-up resistors required proper inversion

LCD Communication: SPI protocol implementation required understanding of 4-bit mode

State Management: Ensuring state transitions occur correctly without conflicts

Overload Recovery: Properly clearing overload state when weight normalizes

7.3 Future Enhancements

Potential improvements to the system:

Emergency Stop Button: Add hardware emergency stop functionality

Position Feedback: Integrate encoder or position sensor for accurate floor detection

Advanced Scheduling: Implement SCAN or LOOK algorithms for multiple requests

Door Control: Add actual door motor control with position feedback

Display Enhancements: Show queue of pending requests on LCD

Energy Efficiency: Implement power-saving modes during idle periods

Multi-Elevator Support: Extend to control multiple elevators

Network Communication: Add serial communication for remote monitoring

7.4 Educational Value

This project successfully demonstrates:

Embedded Systems Concepts: Real-time control, interrupt handling, hardware interfacing

Microcontroller Programming: Direct register manipulation, peripheral configuration

System Design: State machines, modular programming, algorithm design

Hardware Integration: Multiple peripheral interfaces (ADC, PWM, SPI, GPIO)

Safety Engineering: Overload protection, state validation

8. Conclusion

This project successfully implements a functional 3-floor elevator control system on the Freescale HC12 microcontroller platform. The system demonstrates proficiency in embedded systems programming through the integration of multiple hardware peripherals, implementation of a state machine-based control algorithm, and development of safety mechanisms including overload protection.

Key achievements include:

Complete System Integration: Successfully interfaced ADC, PWM, SPI, and GPIO peripherals

Robust Control Logic: Implemented reliable state machine with proper transitions

User Interface: Provided comprehensive visual and audio feedback

Safety Features: Implemented weight-based overload detection and prevention

Efficient Algorithm: Developed priority-based floor selection algorithm

The system meets all specified functional requirements and demonstrates the application of fundamental embedded systems engineering principles. The interrupt-driven architecture ensures responsive operation, while the state machine design provides clear and maintainable code structure.

This project serves as an excellent example of real-world embedded systems application, combining hardware interfacing, real-time control, and user interface design in a cohesive system.

9. References

Freescale Semiconductor. (2005). MC9S12DT256 Device User Guide. NXP Semiconductors.

Freescale Semiconductor. (2003). HCS12 Microcontrollers Reference Manual. NXP Semiconductors.

Hitachi. (1999). HD44780U (LCD-II) Dot Matrix Liquid Crystal Display Controller/Driver Datasheet.

CodeWarrior Development Studio for HC12/S12(X) V5.1 User Guide. (2007). Freescale Semiconductor.

Mazidi, M. A., & Mazidi, J. G. (2006). The 8051 Microcontroller and Embedded Systems Using Assembly and C. Pearson Education.

Valvano, J. W. (2011). Embedded Systems: Introduction to ARM Cortex-M Microcontrollers. Jonathan Valvano.

LaMothe, A. (2003). Tricks of the Windows Game Programming Gurus. Sams Publishing.

Appendix A: Code Structure

A.1 Main Program Flow

Entry → Initialization → MainLoop
                              ↓
                    ┌─────────┴─────────┐
                    ↓                 ↓
            OverloadWait         Movement Check
                    ↓                 ↓
            (Handle Overload)    MOVEUP/MOVEDOWN
                                        ↓
                                   ARRIVED
                                        ↓
                                   MainLoop

A.2 Interrupt Flow

MCCNT_ISR (every 100ms)
    ↓
Read ADC → Check Overload
    ↓
Poll Buttons → Set Requests
    ↓
Select Target (if idle)
    ↓
RTI

A.3 Key Subroutines

init_PWM_Buzzer: Initialize PWM for buzzer control

Buzz: Enable buzzer (set PWME bit)

StopBuzz: Disable buzzer (clear PWME bit)

DoorCloseBeep: Play long beep before movement

SENDINST: Send instruction to LCD

SENDDATA: Send data character to LCD

SENDSPI: Low-level SPI transmission

DELAY: Software delay loop

DelayADC: Delay for ADC stabilization

Appendix B: Hardware Connections

B.1 Port Mapping Summary

| Port | Pin | Function | Direction |
| --- | --- | --- | --- |
| A | 2 | Internal F0 Button | Input |
| A | 3 | Internal F1 Button | Input |
| A | 4 | Internal F2 Button | Input |
| A | 5 | External F0 Button | Input |
| A | 6 | External F1 Button | Input |
| A | 7 | External F2 Button | Input |
| T | 5 | Red LED | Output |
| T | 6 | Yellow LED | Output |
| T | 7 | Green LED | Output |
| P | 4 | Buzzer (PWM) | Output |
| M | - | SPI Interface (LCD) | Output |
| ADC | 5 | Weight Sensor | Input |

B.2 Register Configuration Summary

Port A (Buttons):

DDRA = $00 (all inputs)

PUCR = $02 (pull-up enabled)

Port T (LEDs):

DDRT = %11100000 (PT5-PT7 outputs)

PTT = %00000000 (all LEDs off initially)

PWM (Buzzer):

PWMPRCLK = $03 (prescaler E/8)

PWMPER4 = 250 (period)

PWMDTY4 = 125 (duty cycle 50%)

PWME bit 4 = enable/disable

ADC (Weight Sensor):

ATD0CTL2 = %11000000 (enable, fast flag clear)

ATD0CTL3 = %00001000 (single conversion)

ATD0CTL4 = %10000101 (8-bit, prescaler)

ATD0CTL5 = %10100101 (channel 5, right-justified)

SPI (LCD):

SPI0CR1 = $52 (master mode, CPOL=1, CPHA=0)

SPI0CR2 = $10 (mode fault enable)

SPI0BR = $00 (baud rate)

MCCNT (Interrupt):

MCCTL = %11000111 (enable, interrupt enable)

MCCNT = 6250 (100ms period)

Appendix C: State Transition Diagram

[IDLE]
         /  |  \
        /   |   \
       /    |    \
      /     |     \
     /      |      \
    /       |       \
[UP]    [DOWN]  [OVERLOADED]
  |       |          |
  |       |          |
  └───→ [ARRIVED] ←──┘
         |
         └──→ [IDLE]

Legend:

Solid arrows: Normal transitions

Dashed arrows: Overload transitions

[ARRIVED] is a temporary state that always returns to [IDLE]